NC Fire Response Data Layer information

Date: April 2026

The NHPFireRespon dataset is provided to help fire fighters avoid impacts to sensitive resources when creating fire lines.

For questions or comments, contact Natural Heritage Program: scott.pohlman@ncdcr.gov, misty.buchanan@ncdcr.gov, mitch.east@dncr.nc.gov

color-coded legend

- Red (avoid plowing) - always consult with NHP, regulatory authorities, and/or land managers before plowing.

  - Dedicated Nature Preserve Primary and Special Management Areas;

  - Species occurrences: Federally Endangered, Threatened, Candidate, Proposed Endangered, Proposed Threatened, BGPA;

  - Species or natural community occurrences: G1 (Globally Critically Imperiled) or G2 (Globally Imperiled);

  - Unique Wetlands

- Orange (proceed with caution) - Consult if time allows. In situations where time does not allow for prior consultation, it would be great to be notified after the fact that there was an emergency impact.

  - Dedicated Nature Preserve Primary Restoration Areas;

  - NC Land and Water Fund Easements;

  - Species occurrences: NC Endangered, Threatened, Special Concern, and Significantly Rare;

  - Natural Community occurrences: S1 (NC Critically Imperiled), S2 (NC Imperiled), S3 (NC Vulnerable); Include only records ranked A or B (Excellent or Good viability);

- Yellow (minimize impacts); no need to consult, but minimize footprint of fire lines and use existing breaks where possible;

  - Dedicated Nature Preserve Buffer

  - Natural Community occurrences: S4 (NC -Apparently Secure), S5 (NC Secure); Include only records ranked A or B (Excellent or Good viability);

Attribute Field Names:

dnpbuff1 – DNP buffer zones
dnpprisma3 – DNP primary and SMA zones

dnprest2 – DNP restoration zones

fedeos3 – federally listed EOs

g1g2eos3 – G1 and G2 species EOs

natcoms1 – A/B ranked S4/S5 natural communities

natcoms2 – A/B ranked S1-S3 natural communities

OID - automatically generated primary key/ID field

rhalwf2 – RHAs or LWF easements

stateeos2 – state listed EOs

uw3 – Unique Wetlands